Technion – Israel Institute of Technology

# HW5

Numerical Methods

019003

| Alon Spinner | 305184335 | alonspinner@gmail.com |
| --- | --- | --- |
| Oren Elmakis | 311265516 | orenelmakis@gmail.com |

December 29, 2021

# Question 1 –

The control volume approach is based on assumption of steady state and equilibrium between the fluxes

## Section  a

We were asked to use the volume control approach for calculating the points 1,2,3,4 boundary points. We assume the plate is found in steady state, and first we will write the heat balance as follows:

### Point 1 –

We created a control volume of size Δy1,Δx1. The flux direction of the convection is into the control volume, with intersection Δy1. Accordingly, the conduction flux in the x direction is outward from the cell to cell i+1,j , and the conduction flux in y is moving into the control volume from downward, to cell j+1.

0=hcTa-Ti,jΔy1left side convection+k1Ti+1,j-Ti,jΔx1Δy1right side conduction-k1Ti,j-Ti,j-1Δy1Δx12from downward conduction+k1Ti,j+1-Ti,jΔy1Δx12outward conduction

According to the given grid Δx1=Δy1=Δx, after the reduction, we get

0=hcTa-Ti,jΔy1+k1Ti+1,j-Ti,j-k1Ti,j-Ti,j-12+k1Ti,j-Ti,j-12

Now, we will gather all the variables together and move the variable Ti,j to the right side

Ti,jhcΔy1+2k1=TahcΔy1+k12Ti,j-1+k12Ti,j+1+k1Ti+1,j

### Point 2 –

We created a control volume of size Δy2,Δx2. The flux direction of the convection is into the control volume, with intersection Δx2. Accordingly, the conduction flux in the y direction is outward from the cell to cell i,j-1, and the conduction flux in x is moving into the control volume from the left side cell i-1, to cell i+1.

0=hcTa-Ti,jΔx2from upward convection+k2Ti,j-1-Ti,jΔy2Δx2downward conduction-k2Ti,j-Ti-1,jΔx2Δy22left side flux+k2Ti+1,j-Ti,jΔx2Δy22right side flux

After the reduction, according to the second material grid Δx2=Δx and Δy2=2Δx, we get

0=hcTa-Ti,jΔx2+k22Ti,j-1-Ti,j-k2Ti,j-Ti-1,j+k2Ti+1,j-Ti,j

Now, we will gather all the variables together and move the variable Ti,j to the right side

Ti,jhcΔx2+52k2=TahcΔx2+k22Ti,j-1+k2Ti+1,j+k2Ti-1,j

### Point 3 –

We created a control volume of size Δy1,Δx12+Δx22. The insulated edge preventing the heat flux from outward. Accordingly, the conduction flux in the y direction is outward from the cell to two cells cell i,j+1 , and the conduction flux in x is moving into the control volume from the material 1 in the left side cell j,i-1, to cell j,i+1.

0=k1Ti,j+1-Ti,jΔy1Δx12+k2Ti,j+1-Ti,jΔy1Δx22upward flux -k1Ti,j-Ti-1,jΔx1Δy12left side flux+k2Ti+1,j-Ti,jΔx2Δy12right side flux

The grid is  Δx22+Δx12=Δx,  Δy1=Δx, after the reduction, we get

0=12k1Ti,j+1-Ti,j+12k2Ti,j+1-Ti,j-12k1Ti,j-Ti-1,j+12k2Ti+1,j-Ti,j

Now, we will gather all the variables together and move the variable Ti,j to the right sideTi,jk1+k2=12Ti,j+1k1+k2+k12Ti-1,j+k22Ti+1,j#

### Point 4 –

We created a control volume of size Δx2,Δy2. The insulated edge preventing the heat flux from outward. Accordingly, the conduction flux in the y direction is outward from the cell to two cells cell i,j+1 , and the conduction flux in x is moving into the control volume from the material 1 in the left side cell j,i-1, to cell j,i+1.

0=-k2Ti,j-Ti-1,jΔx2Δy22left side flux+k2Ti+1,j-Ti,jΔx2Δy22right side flux +k2Ti,j+1-Ti,jΔy2Δx2upward flux

According to the second material given grid Δx2=Δx and Δy2=2Δx, after the reduction, we get

0=-Ti,j-Ti-1,j+Ti+1,j-Ti,j +12Ti,j+1-Ti,j #

Now, we will gather all the variables together and move the variable Ti,j to the right side

Ti,j=2Ti-1,j+2Ti+1,j+Ti,j+15

## Section b

We were asked to formulate the equations of points 5,6,7:

### Point 5 –

We created a control volume of size Δy1,Δx12+Δx22. We assume the flux is flows from left to right in direction x, and from downward to upward in direction y.

0=k1Ti,j+1-Ti,jΔy1Δx12+k2Ti,j+1-Ti,jΔy1Δx22upward flux-(k1Ti,j-Ti,j-1Δy1Δx12+k2Ti,j-Ti,j-1Δy1Δx22)from downward flux -k1Ti,j-Ti-1,jΔx1Δy1left side flux+k2Ti+1,j-Ti,jΔx2Δy1right side flux

The grid size is  Δx22+Δx12=Δy12=Δx, after the reduction, we get

0=12k1Ti,j+1-Ti,j+12k2Ti,j+1-Ti,j-12k1Ti,j-Ti,j-1-12k2Ti,j-Ti,j-1 -k1Ti,j-Ti-1,j+k2Ti+1,j-Ti,j #

Now, we will gather all the variables together and move the variable Ti,j to the right side

Ti,jk1+k2=Ti,j+1k1+k2+Ti,j-1k1+k2+2k1Ti-1,j+2k2Ti+1,j4

### Point 6 –

According to the figure of the plate

Tij=T2

### Point 7 –

We created a control volume of size Δy1,Δx12+Δx22. We assume the flux is flows from left to right in direction x, and from downward to upward in direction y.

0=hcTa-Ti,jΔx12+hcTa-Ti,jΔx22upward convection-(k1Ti,j-Ti,j-1Δy1Δx12+k2Ti,j-Ti,j-1Δy1Δx22downward conduction) -k1Ti,j-Ti-1,jΔx1Δy1left side flux+k2Ti+1,j-Ti,jΔx2Δy1right side flux #1

The grid size is  Δx22+Δx12=Δy12=Δx, after the reduction, we get

0=hcTa-Ti,jΔx12+hcTa-Ti,jΔx22-k12Ti,j-Ti,j-1-k22Ti,j-Ti,j-1 -k1Ti,j-Ti-1,j+k2Ti+1,j-Ti,j #2

Now, we will gather all the variables together and move the variable Ti,j to the right side

Ti,jhcΔx12+Δx22+32k1+k2=hcTaΔx12+Δx22+Ti,j-12k1+k2+Ti-1,jk1+

Ti+1,jk2 #3

## Section c

We will open the equation using the finite difference approach using second order central difference.

∂2T∂x2+∂2T∂y2=0→Ti+1,j-2Ti,j+Ti-1,jΔx2+Ti+1,j-2Ti,j+Ti-1,jΔy2=0

The steps of Δx and Δy are equal, thus

∂2T∂x2+∂2T∂y2=0→Ti+1,j-2Ti,j+Ti-1,j+Ti+1,j-2Ti,j+Ti-1,j=0

We can arrange the equation as follows (keep remember that Δx=Δy) in material 1(

Ti+1,j-Ti,jΔx Δy-Ti,j-Ti-1,jΔx Δy+Ti,j+1-Ti,jΔy Δx+Ti,j-Ti,j-1Δy Δx=0

Multiply by k1, and we get the control volume scheme

k1Ti+1,j-Ti,jΔxΔyright side flux-k1Ti,j-Ti-1,jΔxΔyleft side flux+k1Ti,j+1-Ti,jΔy Δxupward flux+k1Ti,j-Ti,j-1Δy Δxfrom downward flux=0

# Question 2 –

## Section a

The equation is given as

∂u∂t=k∂2u∂x2+νx,t

The time first derivative is based on forward differentiation

∂u∂t=uit+1-uitΔt,

Accordingly, the explicit form of the partial second derivative is a upward triangle, with time propagation on the top, for index i

∂2u∂x2=ui+1t-2uit+ui-1tΔx2

uit+1-uitΔt=kui+1t-2uit+ui-1tΔx2+νxi,ti

The implicit representation form is based on inverted triangle,

∂2u∂x2=ui+1t+1-2uit+1+ui-1t+1Δx2

uit+1-uitΔt=kui+1t+1-2uit+1+ui-1t+1Δx2+νxi,ti

The Crank Nicolson method uses dummy point that is in the middle of the time step, and represented as a combination of the time propagation and the former state:

∂2u∂x2=12ui+1t+1-2uit+1+ui-1t+1Δx2+ ui+1t-2uit+ui-1tΔx2

uit+1-uitΔt=12ui+1t+1-2uit+1+ui-1t+1Δx2+ ui+1t-2uit+ui-1tΔx2+νxi,ti

## Section b

We were asked to solve the heat transfer function using the explicit method.

First, we have to formulate the boundary conditions of both sides:

In x=0 according to the conservation of heat equation:

qx-qx+ΔxΔx=ρCTi+1-TiΔt

From Fourier’s law

hcTa-T0i+1+kTj+1i+1-Tji+1Δx+∫vxj,ti+1-vxj+1,ti+1dxdtΔx=Tji+1-TjiΔt

Tji+1Δx+hcΔt+kΔtΔx=TahC+kdxTj+1i+1+∫vxj,ti+1-vxj+1,ti+1dxdtΔt+TjiΔx

In x=1, the outward flux is known q=1

-kTj+1i+1-Tji+1Δx+∫vxj,ti+1-vxj+1,ti+1dxdt-q1Δx=Tji+1-TjiΔt

Tj+1i+1Δx+kΔtΔx=kΔxTji+1+∫vxj,ti+1-vxj+1,ti+1dxdt-q1Δt+TjiΔx

And the central nodes equations for temperature distribution

Tit+1=Tit + kΔtTi+1t-2Tit+Ti-1tΔx2+νxi,tiΔt

The convergence and stability of the explicit method is

Δt≤12Δx2k

As a result using Δx=0.1 and Δt=0.01 causing to diverge.

The following figure demonstrate the time-location distribution of the 1D heated rod with Δx=0.2 and Δt=0.01:

## Section c

The boundary conditions in the same manner as in section a:

Tji+1Δx+hcΔt+kΔtΔx=TahC+kdxTj+1i+1+∫vxj,ti+1-vxj+1,ti+1dxdtΔt+TjiΔx

However now we don’t have the Tj+1i+1

Tji+1Δx+hcΔt+kΔtΔx-kΔtΔxTj+1i+1=TahC+∫vxj,ti+1-vxj+1,ti+1dxdtΔt+TjiΔx

And the boundary condition of the end of the rod:

Tj+1i+1Δx+kΔtΔx-kΔtΔxTji+1=∫vxj,ti+1-vxj+1,ti+1dxdt-q1Δt+TjiΔx

The central nodes equation is:

uit+1=uit+Δtk2ui+1t+1-2uit+1+ui-1t+1Δx2+ ui+1t-2uit+ui-1tΔx2+νxi,ti

The following figure demonstrate the time-location distribution of the 1D heated rod with Δx=0.2 and Δt=0.01:

## Section d

In this question we were asked to implement the methods of explicit and crank Nicolson for solving the 1D heated rod pde. The main difference between the methods are the performing complexity and the stability. In compare to the Crank Nicolson which requires tridiagonal algebraic solution the explicit method implementation is straight forward. However, in contrast to the Crank Nicolson method that has a unconditionally stability the explicit approach is restricted to stability term of Δt≤12Δx2k.

# Question 3 –

## Section a

Given that dx=dy we have to prove the ADI scheme for inner points, the PDE equation for plate 2x2

∂T∂t=k∂2T∂x2+∂2T∂y2+fx,yT

The first step is forward differentiation of Δt2

∂T∂t=Ti,jl+12-Ti,jlΔt2

time propagation of Δt2 in the y direction

∂2T∂y2=Ti,j+1l+12-2Ti,jl+12+Ti,j-1l+12Δy2

And using second order central difference OF x explicitly

∂2T∂y2=Ti+1,jl-2Ti,jl+Ti-1,jlΔx2

Consequently, with Δx=Δy and define λ=ΔtkΔx2 the results

2Ti,jl+12-2Ti,jl=λTi+1,jl-2Ti,jl+Ti-1,jl+Ti,j+1l+12-2Ti,jl+12+Ti,j-1l+12+Δtfx,yTi,jl

after some

-λTi,j-1l+12+21+λTi,jl+12-λTi,j+1l+12=λTi-1,jl+21-λTi,jl+λTi+1,jl+Δtfx,yTi,jl

The second step from l+Δt2→l+Δt

∂T∂t=Ti,jl+1-Ti,jl+12Δt2

time propagation of Δt in the x direction

∂2T∂x2=Ti,j+1l+1-2Ti,jl+1+Ti,j-1l+1Δx2

With y remaining the same.

∂2T∂y2=Ti,j+1l+12-2Ti,jl+12+Ti,j-1l+12Δy2

Consequently, with Δx=Δy and define λ=ΔtkΔx2 the results

2Ti,jl+1-2Ti,jl+12=λTi+1,jl+1-2Ti,jl+1+Ti-1,jl+1+Ti,j+1l+12-2Ti,jl+12+Ti,j-1l+12+Δtfx,yTi,jl+0.5

And after order

-λTi-1,jl+1+21+λTi,jl+1-λTi+1,jl+1=λTi,j-1l+0.5+21-λTi,jl+0.5+λTi,j+1l+0.5+Δtfx,yTi,jl+0.5

## Section b

The initial condition is

Tx,y,0=0

With the initial condition we got three trivial boundary conditions

Tx,0,t=x2-x

T2,y,t=2-y

Tx,2,t=1

And another Neumann condition

∂T0,y,t∂x=0

We will use dummy boundary condition, outside the domain (to keep the accuracy of the method, using central derivative)

T-1,yt-T1,yt2Δx=0

T-1,yt=T1,yt

Now, we can substitute to  the ADI equation

-λT0,j-1l+12+21+λT0,jl+12-λT0,j+1l+12=λT1,jl+21-λT0,jl+λT1,jl+Δtf0,yT0,jl

-λT0,j-1l+12+21+λT0,jl+12-λT0,j+1l+12=21-λT0,jl+2λT1,jl+Δtf0,yT0,jl

-λT1,jl+1+21+λT0,jl+1-λT1,jl+1=λT0,j-1l+0.5+21-λT0,jl+0.5+λT0,j+1l+0.5+Δtfx,yT0,jl+0.5

21+λT0,jl+1-2λT1,jl+1=λT0,j-1l+0.5+21-λT0,jl+0.5+λT0,j+1l+0.5+Δtfx,yT0,jl+0.5

## Section d

According to λ=16 we rounded the Δt to 0.001 seconds.